# Touch Screen Equipment

Figure 2-1 Connector Layout for the TSE

J1 Connector type: 31 pin socket double density D subminiature P/N 2DA31S-FO (ITT Cannon) plus contacts 030-9542-012 (ITT CANNON).  Suggested Mating connector:  P/N 2DA31P-FO (ITT Cannon) plus contacts 030-9540-000 (ITT CANNON),  and Backshell P/N K3047 (KERN ENGINEERING).

Table 2-1  TSE I/O Connector

| In/Out | TSE Signal Name | Description | Connector | Pin | Usage: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | A | B | C | F | G | M |
| In | +28VDC | DC Power (PIN) | J1 | 13 | X | X | X | X | X | X |
| In | Address | LRU address 4 | J1 | 21 |  |  |  |  |  | X |
|  | 28V_RTN | DC Power RTN (SOCKET) | J1 | 14 | X | X | X | X | X | X |
| In | RS232_MUX_SEL_0 | RS232 MUX SELECT | J1 | 20 |  |  |  |  |  | X |
| In | NTSC_COMP_IN | NTSC Composite Signal In | J1 | 10 |  | X | X |  |  | X |
|  | NTSC_COMP_RTN | NTSC Composite Signal In | J1 | 31 |  | X | X |  |  | X |
| In | AUDIO_IN_L | Audio In | J1 | 7 |  | X | X |  |  | X |
| In | AUDIO_IN_R | Audio In | J1 | 18 |  | X | X |  |  | X |
|  | AUDIO_IN_RTN | Audio In RTN | J1 | 28 |  | X | X |  |  | X |
| Out | AUDIO_OUT_L | Audio Out | J1 | 8 |  |  |  |  | X | X |
| Out | AUDIO_OUT_R | Audio Out | J1 | 19 |  |  |  |  | X | X |
|  | AUDIO_OUT_RTN | Audio Out RTN | J1 | 29 |  |  |  |  | X | X |
| Out | RS232_TX | RS232 | J1 | 5 | X | X | X |  |  | X |
| In | RS232_RX | RS232 | J1 | 26 | X | X | X |  |  | X |
|  | RS232_RTN | RS232 RTN | J1 | 16 | X | X | X |  |  | X |
| Out | ETHERNET_TX_H | 10/100Base-T Ethernet Transmit Data + | J1 | 11 | X | X | X | X | X | X |
| Out | ETHERNET_TX_L | 10/100Base-T Ethernet Transmit Data - | J1 | 12 | X | X | X | X | X | X |
| In | ETHERNET_RX_H | 10/100Base-T Ethernet Receive Data + | J1 | 1 | X | X | X | X | X | X |
| In | ETHERNET_RX_L | 10/100Base-T Ethernet Receive Data - | J1 | 22 | X | X | X | X | X | X |
|  | Address | LRU address 3 | J1 | 2 | X | X | X | X | X | X |
|  | GND | GND REFERENCE | J1 | 23 | X | X | X | X | X | X |
| In | ADDRESS/JTAG_SELECT | ADDRESS/JTAG_SELECT | J1 | 9 |  |  |  |  |  | X |
| In | TDI_/_LRU_ADDRESS_0_LSB | JTAG / LRU ADDRESS 0 LSB | J1 | 30 |  |  |  | X | X | X |
| In | TDO_/_LRU_ADDRESS_1 | JTAG / LRU ADDRESS 1 | J1 | 15 |  |  |  | X | X | X |
| In | TCK_/_LRU_ADDRESS_2 | JTAG / LRU ADDRESS 2 | J1 | 17 |  |  |  | X | X | X |
| In | TMS_/ LRU Address Parity | JTAG/ LRU Address Parity | J1 | 27 |  |  |  | X | X | X |
| In | RESET/TRST | AUX RESET | J1 | 6 |  |  |  |  |  | X |
| In/Out | D+ | USB Data+ | J1 | 3 |  |  | X |  |  | X |
| In/Out | D- | USB Data- | J1 | 24 |  |  | X |  |  | X |
| Out | +5Vdc | USB Power out 5 V at 100 mA | J1 | 4 |  |  | X |  |  | X |
|  | GND | USB Ground | J1 | 25 |  |  | X |  |  | X |

Note: Ground Reference pins are provided for grounding input select, mode and address pins. They may also be used to provide ground reference connections to external devices, but shall not be used to sink/source more than 10 milliamps.

Note: All shields should be terminated to backshell, not to available groundlines.

* When TSE is connected to a ZDE, all address lines must be open.

** Address/JTAG_SELECT is always open in aircraft installation.

***Usage:

A – TSE display in ZDE attached configuration without Video Input or USB

B – TSE display in ZDE attached configuration with Video Input, no USB

C – TSE display in ZDE attached configuration with Video Input and USB

F – Flight Deck

G – Galley Display

M – Bench Maintenance